# Related Works

## Agrivoltaic Systems: Architecture, Crop Response, and Land-Use Efficiency

The concept of co-locating solar panels and crops on the same land parcel was first proposed by , who noted that partial shade from PV modules could reduce transpiration stress while the modules themselves benefited from evaporative cooling. The idea remained largely theoretical until  published the first rigorous field experiment and introduced the Land Equivalent Ratio (LER) as the standard metric for the dual land-use advantage. Measuring durum wheat yield and PV output simultaneously in southern France, they demonstrated LER values of 1.35–1.73 depending on shade level, establishing the foundational result that the combined system outperforms two separate monocultures by 35–73% on the same land area.

Subsequent field studies confirmed that the benefit extends beyond electricity and yield.  demonstrated that the microclimate under APV panels — characterised by soil temperature reductions of 1.3–2.3 ​∘C and reduced peak radiation load — can partially offset or reverse the yield penalty of reduced PAR for shade-adapted lettuce, cucumber, and wheat. In semi-arid Arizona,  showed that agrivoltaics simultaneously increased jalapeno yield by 3×, reduced water consumption by 65%, and improved panel efficiency by 1–9 percentage points through evaporative cooling, providing the first comprehensive food-energy-water nexus evidence for the technology.

The integration of bifacial modules — which harvest both direct front-side and diffuse rear-side irradiance — represents a significant performance advance.  developed a Light-Productivity-Factor (LPF) framework that accounts for the PAR saturation threshold of the specific crop. Applied to bifacial tomato production in Arizona, this approach yielded the highest published two-component LER of 1.94, demonstrating that crop-specific geometry optimization can push the land-use advantage substantially beyond geometry-agnostic designs. The PAR saturation concept introduced by  — the observation that above a crop-specific irradiance threshold, additional light generates no additional photosynthesis and the surplus can be productively captured as electricity — forms the theoretical basis for the crop yield model adopted in the present study.

extended this approach to open-field lettuce in Italy, achieving LER = 1.85 under optimised panel density and identifying the circular reference problem in LER​PV calculation: using the same APV array as both system and reference inflates the LER value artificially. This issue motivates the independent dense-pack PV reference (Definition A) adopted here. At the system scale, a meta-analysis of 127 agrivoltaic case studies by  found a mean LER of 1.34±0.18, with higher values consistently associated with bifacial modules, shade-tolerant crops, and optimised row spacing. Water savings from APV shading, quantified by  across a lettuce trial in southern France, reached 14–29% of growing-season ET​0 — a range consistent with the corrected values reported in the present study.

Despite this body of evidence, all cited agrivoltaic studies share a common limitation: the crop residue stream is treated as waste or a soil amendment, severing the link between the agrivoltaic system and the biogas economy that the same semi-arid farming regions are increasingly participating in.

## Anaerobic Digestion: Fundamentals, Feedstocks, and Thermal Management

Anaerobic digestion is a multi-stage biological process in which consortia of microorganisms decompose organic matter in the absence of oxygen, producing biogas — typically 55–70% methane and 30–45% CO​2 — and a nutrient-rich liquid digestate . The four sequential stages — hydrolysis, acidogenesis, acetogenesis, and methanogenesis — each involve distinct microbial communities with different kinetic sensitivities to temperature, pH, and substrate composition . This sequential dependency makes methanogenesis the rate-limiting step and temperature the dominant operational parameter: a 10 ​∘C decrease in digester temperature below the mesophilic optimum (37 ​∘C) can suppress methane yield by 30–50%, following an Arrhenius-type relationship .

The biochemical methane potential (BMP) of a substrate quantifies the maximum volume of methane recoverable per unit of volatile solids (VS) under controlled conditions.  reported BMP values of 200–280 NmL CH​4 gVS​−1 for cattle manure and 250–350 NmL CH​4 gVS​−1 for tomato plant residues — ranges that bracket the reference value of 300 NmL CH​4 gVS​−1 at 37 ​∘C adopted in the present study.  provided a five-point temperature response curve for tomato plant residues from 20 ​∘C to 40 ​∘C, demonstrating that BMP drops from 300 NmL gVS​−1 at 37 ​∘C to approximately 195 NmL gVS​−1 at 20 ​∘C — a 35% suppression that is directly relevant to cold-season operation at Konya and Freiburg. This dataset provides the empirical basis for the V2 model validation reported in Section [sec:results].  extended this analysis to cereal straw and vegetable mixtures, confirming the Arrhenius coefficient θ=0.069 ​∘C​−1 used throughout the present study and reporting that co-digestion of crop residues with cattle manure at a 1:3 VS ratio increases BMP by 12–18% through synergistic microbial diversity.

The thermal energy demand of a continuously stirred tank reactor (CSTR) digester operating under mesophilic conditions in a temperate climate is substantial.  showed that maintaining a 37 ​∘C digester against a 0 ​∘C ambient requires heat input equivalent to 15–30% of gross biogas energy, depending on insulation quality and substrate inlet temperature. This self-consumption directly reduces the net energy yield and economic return of the plant. Solar thermal collectors  and PV-powered electrical resistance heating  have both been proposed as alternatives to biogas self-consumption.  demonstrated that replacing biogas self-heating with PV electricity in a grid-connected configuration can improve the NPV of a biogas plant by up to 8.7× compared to a non-heated reference, by redirecting the saved biogas to higher-value grid injection. Their analysis, however, treats the PV array as a separate installation without modelling the agrivoltaic geometry or crop yield effects.

On the feedstock supply side, the strategic importance of agricultural biogas and biomethane is reflected in accelerating policy commitments. The EU REPowerEU Plan targets 35 billion m​3 of sustainable biomethane per year by 2030 , requiring a nine-fold increase from 2022 levels. Germany’s EEG 2023 and the EU RED III mandate feed-in premiums for biomethane grid injection, creating a market price approximately 5.5 times the wholesale electricity price in Central European markets . In Sub-Saharan Africa, where Burkina Faso’s national grid relies on 63% diesel generation, the IEA projects biogas from agricultural residues as a priority pathway for rural electrification and LPG substitution by 2030 . These policy contexts directly motivate the site-specific biomethane premium scenarios (S5) and carbon credit scenarios (S6) evaluated in the present study.

## PV–Biogas Hybrid Systems: Architecture and Economics

The complementarity between solar PV and biogas is thermodynamic as much as economic. PV generation is intermittent and peaks in summer, while biogas is dispatchable and its heating demand peaks in winter — a seasonal anti-correlation that makes the two technologies natural partners in a hybrid energy system . Early hybrid configurations focused on off-grid rural electrification, combining a biogas generator for baseload with PV for daytime peak demand and battery storage for overnight supply. More recently, grid-connected configurations that exploit feed-in premiums for both electricity and biomethane have become an important economic model in European markets .

Techno-economic analyses of PV–biogas hybrids consistently report that the optimal configuration depends critically on the local electricity tariff structure and the relative prices of grid electricity, LPG, and biomethane.  showed that in markets where grid electricity is cheap relative to biogas, using PV electricity to heat the digester (rather than burning biogas for self-heating) frees the entire biogas output for sale, improving IRR by 3–8 percentage points.  demonstrated that a heat pump powered by APV electricity can maintain thermophilic digestion (52 ​∘C) year-round in a continental climate, achieving full thermal self-sufficiency with a coefficient of performance (COP) of 3.2–4.1, equivalent to a 70% reduction in heating energy relative to direct resistance heating.

Multi-generation system analyses — simultaneously producing electricity, heat, biomethane, and digestate fertilizer — have been explored by , who reported that joint optimization of all four output streams increases system NPV by 22–34% compared to single-output optimization. Their framework, however, does not include a land-use efficiency metric or crop yield modelling, treating the agricultural component only as a feedstock source.

A critical gap in all cited PV–biogas studies is the absence of agrivoltaic geometry modelling: the PV array is treated as an independent installation with no interaction with the underlying crop canopy. This simplification ignores the fact that in an integrated APV–AD system, the panel geometry simultaneously determines PV yield, crop PAR availability, residue production, and VS input to the digester — four interlinked variables that must be co-optimized rather than treated sequentially.

## The Land Equivalent Ratio: History, Extensions, and Limitations

The Land Equivalent Ratio was introduced by  for intercropping systems, where it quantifies the relative land area under separate monocultures that would be required to produce the same combined yields as an intercropped system. An LER of 1.4, for example, means that 40% more land would be needed under monoculture to achieve the same total output — a powerful argument for integration in land-constrained contexts.  adapted the metric to agrivoltaics by replacing one of the two intercrop outputs with PV electricity, defining the two-component agrivoltaic LER as:

LER2C=Ycrop,APVYcrop,open+EPV,APVEPV,ref

where the denominators are the yields of the same crop and PV technology operated separately on equivalent land areas.

The choice of reference system for the PV denominator has emerged as a methodological fault line in the agrivoltaic literature. Three definitions are in common use, each yielding substantially different numerical LER values for the same physical system. The first — used by  and most early studies — compares APV electricity output to a dense-pack ground-mount array at the same tilt, giving a rigorous independent reference (Definition A in the present study). The second compares the APV output to the same APV array itself, making LER​PV equal to the Ground Coverage Ratio by definition — a circular reference identified and criticised by . The third normalizes by actual irradiance received rather than potential output, conflating system performance with resource availability.  showed that the choice between Definition A and the circular definition changes reported LER values by 0.15–0.35, making cross-study comparisons unreliable unless the reference is stated explicitly. The present study reports results under both Definition A (primary) and Definition B (circular, for literature comparability), and flags the numerical difference explicitly in the Results section.

A second unresolved limitation of the classical LER framework is its restriction to two outputs. In an integrated APV–AD system, three distinct land-use outputs are produced simultaneously: crop yield, PV electricity, and biogas energy. Collapsing the third output into the economic analysis — as a revenue item in an NPV calculation — obscures its contribution to land-use efficiency and prevents direct comparison with classical LER values. No prior study has proposed a three-component LER formulation that preserves the dimensional consistency and interpretive clarity of the original metric while accommodating a biogas energy term. The eLER proposed in the present study fills this gap, defining LERbiogas as the ratio of APV-system biogas output to the output of a standalone AD unit processing the same land area under open-field conditions, and summing all three components into a single dimensionless indicator of multi-output land productivity.

## The APV–AD Frontier: First Integration Studies

An early study to explicitly couple agrivoltaic and anaerobic digestion optimization was , who applied a techno-economic framework to a grid-connected APV–AD plant in northern Italy. Their system integrates single-axis tracking PV panels above an agricultural field, with biogas produced from on-site biomass feeding a combined heat and power (CHP) unit or a biomethane upgrading system for grid injection. Using NPV as the objective function and land occupation as a secondary constraint, they found that on-grid configurations with heat pump integration require 17–20 times less land than off-grid alternatives and achieve NPV values up to million at the optimised design. The biomethane grid injection scenario, valued at 5.5 times the electricity purchase price under EEG 2023 and RED III incentives, emerged as the economically preferred valorisation strategy — a finding that directly motivates Scenario S5 of the present study.

Despite its pioneering contribution, the work of  has four significant limitations that the present study addresses. First, the analysis is geographically limited to a single Mediterranean location (northern Italy, Cfb climate), preventing any inference about how the APV–AD synergy varies with solar resource, ambient temperature, or growing-season length. Second, the crop beneath the panels is treated exclusively as a biomass feedstock source: no crop yield model is implemented, and the effect of panel shading on photosynthesis, PAR distribution, or harvest quantity is not quantified. Third, no land-use efficiency metric is defined beyond raw land occupation; the eLER concept — or any equivalent multi-output indicator — is absent, making it impossible to assess how efficiently the integrated system uses land relative to three separate monocultures. Fourth, the optimization searches only the economic design space (CHP vs. biomethane, on-grid vs. off-grid) without jointly optimizing the APV geometry (tilt, row spacing, module height) and AD design (digester volume, HRT, heating fraction) simultaneously.

Table 1 summarises the positioning of the present study relative to the most relevant prior works across the agrivoltaic, AD integration, and LER literatures.

Positioning of the present study relative to key prior works.  = fully addressed; ∘ = partially addressed; × = not addressed.

| Study | Multi- | Crop | AD | Joint | LER | 4E |
| --- | --- | --- | --- | --- | --- | --- |
|  | climate | yield | thermal | optim. | metric | analysis |
|  | × |  | × | × |  | × |
|  | × |  | × | × |  | × |
|  | × |  | × | ∘ |  | × |
|  | ∘ | × | × | × | × | × |
|  | × | × |  | × | × | ∘ |
|  | × | × |  | × | × | ∘ |
|  | × | × |  | ∘ | × | ∘ |
| This study |  |  |  |  |  |  |

## Multi-Climate Comparative Studies and Research Positioning

Multi-site comparative analyses in agrivoltaic research remain rare.  modelled shading factors and POA irradiance across five European locations using a validated geometric model, but without crop yield, economic, or water analysis. Their study confirmed that the shading geometry is strongly latitude-dependent — the same panel configuration produces significantly different ground irradiance distributions at 38°N (Spain) versus 52°N (Netherlands) — motivating the multi-latitude design of the present study.  conducted a meta-analysis across 127 agrivoltaic case studies spanning Europe, Asia, and North America, finding that climate zone explains approximately 18% of LER variance after controlling for crop species and panel technology, but without isolating the mechanism.

No prior study has applied an integrated APV–AD framework to more than one climate zone, nor has any study included a site from tropical Sub-Saharan Africa (BSh tropical), where the combination of year-round near-optimal digester temperatures (ambient T​amb≥ 22 ​∘C throughout the year), high solar irradiance (2050 kWh   m−2), and acute energy poverty creates a particularly strong case for integrated APV–AD deployment. The inclusion of Ouagadougou, Burkina Faso in the present study — alongside Konya (BSk), Almería (BSh hot), and Freiburg (Cfb) — enables a quantitative comparison of APV–AD performance across the full semi-arid-to-temperate spectrum.

The present study extends prior work by: (i) applying a joint APV–AD optimizer across four climate zones, (ii) introducing a three-component eLER as the optimization objective, (iii) identifying and mechanistically explaining a non-monotone relationship between annual solar resource and eLER through the PAR-saturation mechanism, and (iv) providing a 4E analysis — Energetic, Economic, Energo-economic, and Environmental — for an integrated APV–AD system across contrasting climate zones. Together, these contributions address the six research gaps identified in Section [sec:intro] and establish a replicable framework for the site-specific deployment assessment of integrated APV–AD systems in food-energy-water nexus planning.