# Methodology

## Study Sites and Climate Data

Four sites were selected to span four distinct Köppen–Geiger climate subtypes across a latitudinal range of 12 °N to 48 °N (Table 1, Figure 1). The selection criterion was maximum climate diversity within the semi-arid and adjacent temperate zones that represent the primary deployment contexts for agrivoltaic systems: two hot and cold semi-arid sites (Almería BSh and Konya BSk), one tropical semi-arid site (Ouagadougou BSh), and one oceanic temperate site (Freiburg Cfb) included as a northern European reference.

Location map of the four study sites overlaid on the Köppen–Geiger climate classification. Semi-arid zones (BS) are shaded in light yellow; the oceanic temperate zone (Cfb) in light blue.

Hourly climate data were obtained from the PVGIS-SARAH3 Typical Meteorological Year (TMY) database of the European Commission Joint Research Centre . The SARAH3 satellite product provides direct normal irradiance (DNI) and diffuse horizontal irradiance (DHI) as independent retrievals, without decomposition from global horizontal irradiance (GHI). This avoids the systematic decomposition errors of up to ±8% reported for empirical decomposition models under broken cloud conditions . The mean bias error of PVGIS-SARAH3 GHI estimates is below ±5% for monthly averages at all four study sites . The TMY file format provides ten meteorological variables at hourly resolution (8760 rows per site): air temperature T​2m (°C), relative humidity RH (%), GHI, DNI, DHI (W m​−2), downwelling infrared irradiance IR(h) (W m​−2), 10-m wind speed WS​10m (m s​−1), wind direction WD​10m (°), and surface pressure SP (Pa).

Geographical and climatic characteristics of the four study sites (PVGIS-SARAH3 TMY data).

| Parameter | Konya | Almería | Ouagadougou | Freiburg |
| --- | --- | --- | --- | --- |
| Latitude | 37.87°N | 36.83°N | 12.37°N | 47.99°N |
| Longitude | 32.49°E | 2.46°W | 1.52°W | 7.85°E |
| Elevation (m a.s.l.) | 1 016 | 20 | 303 | 278 |
| Köppen class | BSk | BSh | BSh | Cfb |
| Annual GHI (kWh m​−2) | 1 650 | 1 850 | 2 050 | ∼1 150 |
| Mean annual T (°C) | 11.5 | 18.5 | 28.5 | ∼11.4 |
| Summer T​max (°C) | 38 | 35 | 41 | ∼35 |
| Winter T​min (°C) | −10 | +5 | +15 | ∼−12 |
| Annual ET​0 (mm) | 1 200 | 1 350 | 1 800 | ∼900 |
| Annual precip. (mm) | 320 | 200 | 800 | ∼800 |

Monthly climate profiles for the four study sites from PVGIS-SARAH3 TMY data. Upper row: monthly GHI (kWh m​−2) with annual total annotated; lower row: mean monthly air temperature (°C). Shaded bands indicate the tomato growing season at each site.

The four sites span a mean annual temperature range of 11.4–28.5 °C, a GHI range of 1 150–2 050 kWh m​−2, and growing seasons of 122–189 days. This diversity ensures that the model is tested across the full range of Arrhenius BMP suppression (from negligible at Ouagadougou to severe at Freiburg in January) and PAR saturation regimes (from frequent saturation at Ouagadougou to rare saturation at Freiburg), providing the basis for the multi-climate findings reported in Section [sec:results].

## Crop Parameters and Growing Seasons

Tomato (Solanum lycopersicum) was selected as the reference crop for all four sites on the basis of three criteria: it is cultivated commercially in all four study regions, its PAR saturation point (174 W   m−2) is well-documented and places it in the high-irradiance sensitivity range where APV shading effects are most pronounced, and its residue-to-fruit ratio and BMP are among the best-characterised in the agrivoltaic and AD literatures .

Site-specific planting and harvest dates follow local agricultural calendars (Table 2). The three semi-arid sites use a fresh fruit yield of 60 t   ha−1, consistent with irrigated greenhouse and open-field production in Mediterranean and tropical semi-arid contexts . Freiburg uses 30 t   ha−1 following German open-field production statistics .

Site-specific agronomic, economic, and environmental parameters. Grid emission factors (EF) from  and Umweltbundesamt (2023). Carbon prices from EU ETS (2024) for Almería and Freiburg; voluntary market rates for Konya and Ouagadougou.

| Parameter | Konya | Almería | Ouagadougou | Freiburg |
| --- | --- | --- | --- | --- |
| Planting date | 25 Apr | 1 Mar | 1 Jul | 1 May |
| Harvest date | 30 Oct | 30 Jun | 30 Oct | 15 Oct |
| Season length (days) | 189 | 122 | 122 | 167 |
| Fruit yield (t ha​−1) | 60 | 60 | 60 | 30 |
| Season ET​0 (mm) | 650 | 480 | 620 | ∼350 |
| Residue ratio Rres | 0.80 |  |  |  |
| Dry matter DM (tomato residues) | 0.06 |  |  |  |
| VS fraction VSf | 0.85 |  |  |  |
| Manure input (t ha​−1 yr​−1) | 10.0 |  |  |  |
| Manure DMm | 0.20 |  |  |  |
| Manure VSm | 0.80 |  |  |  |
| Tomato price ($ t​−1) | 300 | 400 | 250 | 350 |
| Electricity price ($ kWh​−1) | 0.06 | 0.09 | 0.12 | 0.09 |
| Discount rate (%) | 8 | 6 | 10 | 5 |
| Grid EF (tCO​2 MWh​−1) | 0.438 | 0.167 | 0.632 | 0.380 |
| Carbon price ($ tCO​2−1) | 15 | 65 | 12 | 65 |

The feedstock to the anaerobic digester combines tomato crop residues and cattle manure as a co-substrate. Manure co-digestion at a VS ratio of approximately 1:2 (residues:manure) improves buffer capacity, microbial diversity, and BMP by 12–18% compared to mono-digestion of crop residues alone . The 10 t   ha−1 fresh manure input corresponds to the production of approximately 2–3 cattle heads per hectare, consistent with the mixed crop-livestock farming systems prevalent in all four study regions. Total annual VS input per hectare is:

VStotal=Yf⋅LERcrop⋅Rres⋅DM⋅VSf⏟crop residues+ Mm⋅DMm⋅VSm⏟cattle manure

where Yf is the fresh fruit yield (kg ha​−1), LERcrop scales the actual residue production under APV shading, and Mm is the fresh manure input (kg ha​−1 yr​−1). At the optimised designs reported in Section [sec:results], VS inputs range from 1892 to 2573 kg   ha−1   yr−1 across the four sites, with Freiburg’s lower fruit yield driving the minimum.

## Solar Geometry and Shading Model

Accurate hourly solar position calculation underpins both the PV transposition model and the inter-row shading factor. Solar declination δ is computed using the Spencer (1971) harmonic approximation:

δ=180π(0.006918−0.399912cosB+0.070257sinB−0.006758cos2B+0.000907sin2B−0.002697cos3B+0.001480sin3B)

where B=2π(n−1)/365 and n is the day of year. The hour angle ω is computed from true solar time, applying a longitude correction and the Equation of Time. The solar elevation angle αs at each hourly timestep is:

sinαs=sinϕsinδ+cosϕcosδcosω

where ϕ is the site latitude.

The Ground Coverage Ratio (GCR) quantifies the fraction of ground area covered by the projected module footprint:

GCR=Lmcosβdrow

where Lm=2.278 m is the module length (Jinko Tiger Neo 550 W​p) and β is the panel tilt angle. A physical non-overlap constraint prevents adjacent rows from casting permanent shade on each other at any sun angle:

drow≥Lmcosβ+0.5 m

where the 0.5 m clearance accounts for module frame width and structural tolerance. This constraint is enforced as a feasibility penalty in the optimizer (Section 1.9).

The hourly ground shading fraction Fshad(t) — the fraction of ground area beneath the array that is shaded at time t — follows :

Fshad(t)=min​1, HmcosΔγdrowtanαs(t)

where Hm is the module bottom-edge height above ground (m), Δγ is the azimuth difference between the solar direction and the row orientation (zero for south-facing rows, which is the configuration adopted throughout), and αs(t) is the solar elevation angle. Fshad=0 at night (αs≤0) and during high-sun periods when the solar elevation is sufficient to eliminate inter-row shading entirely. The shading fraction directly enters both the crop yield model (Section 1.5) and the ET reduction model (Section 1.6), propagating the geometric design variables into all biological and energy outputs of the coupled system.

## Photovoltaic Energy Model

### Plane-of-Array Irradiance

Plane-of-array (POA) irradiance on the tilted front surface is computed using the isotropic sky model of , which decomposes incoming radiation into beam, diffuse, and ground-reflected components:

Gfront=DNIcosθi+DHI1+cosβ2+GHI ρg1−cosβ2

where θi is the angle of incidence on the tilted surface, β is the panel tilt angle, and ρg=0.25 is the ground albedo . The angle of incidence for a fixed, due-south-facing tilted surface is given by the closed-form expression of , expressed directly in terms of declination δ, latitude ϕ, hour angle ω, and tilt β:

cosθi=sinδsin(ϕ−β)+cosδcosωcos(ϕ−β)

This expression is the closed-form relation used here for a fixed due-south collector at every hour of the day. An earlier version of this model used the solar-noon-only approximation cosθi≈sinαscosβ+cosαssinβ, which implicitly assumes the sun remains due south throughout the day; this systematically overestimated front-surface beam irradiance away from solar noon and has been corrected throughout the present study.

For bifacial modules, rear-side irradiance Grear is estimated from ground-reflected radiation modulated by the inter-row shading fraction:

Grear=GHI ρg1−Fshad(t) frear

where frear=0.12 is the rear irradiance fraction relative to ground-reflected GHI, accounting for the view factor from the rear surface to the ground beneath the array . The effective bifacial irradiance combining both surfaces is:

Geff=Gfront+ϕ Grear

where ϕ=0.70 is the bifaciality factor of the Jinko Tiger Neo 550 W​p module.

### Cell Temperature

Cell operating temperature is modelled using the Faiman  approach, which accounts for wind-dependent convective cooling:

Tcell(t)=Tamb(t)+Geff(t)U0+U1 u(t)

where U0=25 W m−2 K−1 is the free convection coefficient and U1=6.84 W s m−3 K−1 is the wind-dependent convective coefficient . Wind speed u(t) is taken directly from the PVGIS-SARAH3 TMY column WS​10m and clipped to a minimum of 0.5 m   s−1 to prevent division by zero in calm conditions.

### PV Power Output

Array power output at each hourly timestep is:

PPV(t)=PSTC N (1−ηloss)1+βpTcell(t)−25Geff(t)1000

where PSTC=550 Wp is the rated module power at Standard Test Conditions, N is the number of modules per hectare (determined by β and drow via Equation [eq:gcr]), ηloss=0.12 aggregates all system losses (wiring, soiling, mismatch, inverter), and βp=−0.0035 K−1 is the temperature power coefficient . Annual PV energy is partitioned between grid export and digester heating:

EPV,heat&=fPV,heat×EPV,totalEPV,sold&=(1−fPV,heat)×EPV,total

where fPV,heat∈[0.05, 0.50] is the sixth design variable optimised by the DE algorithm. Table 3 summarises all fixed PV model parameters.

Fixed PV model parameters.

| Parameter | Symbol | Value | Source |
| --- | --- | --- | --- |
| Rated power (STC) | PSTC | 550 W​p |  |
| Module length | Lm | 2.278 m |  |
| Bifaciality factor | ϕ | 0.70 |  |
| Temp. coefficient | βp | −0.0035 K​−1 |  |
| System losses | ηloss | 0.12 |  |
| Ground albedo | ρg | 0.25 |  |
| Faiman U0 | U0 | 25 W m​−2 K​−1 |  |
| Faiman U1 | U1 | 6.84 W s m​−3 K​−1 |  |
| Rear irradiance fraction | frear | 0.12 |  |

## Crop Yield Model

Relative crop yield is modelled using the PAR integral method, which integrates the gap between available and saturating photosynthetically active radiation over the growing season. The fraction of global horizontal irradiance that is photosynthetically active is fPAR=0.48 . Hourly PAR under the APV array and in an equivalent open field are:

PARAV(t)&=GHI(t)×fPAR×1−Fshad(t)PARopen(t)&=GHI(t)×fPAR

The tomato light saturation point is PARsat=174 W m−2 , above which additional PAR delivers no incremental photosynthesis. The annual yield ratio, integrated exclusively over the growing season [t0, t1], is:

LERcrop=t0t1min​PARAV(t), PARsat dtt0t1PARopen(t) dt

Three properties of Equation [eq:lercrop] are worth noting explicitly. First, LERcrop is bounded above by 1.0: the APV array cannot increase crop yield beyond the open-field reference in the absence of heat stress or water limitation effects, which are not modelled here. Second, at high irradiance sites where PARopen(t)≫PARsat for a large fraction of daytime hours, the denominator is large relative to the numerator even before any shading is applied, making LERcrop sensitive to the ratio PARsat/PARopen — the mechanism that drives the non-monotone eLER–GHI relationship identified in Section [sec:results]. Third, integration is performed over the growing season only: including off-season hours (when GHI≈0 at night and both integrals are zero) does not change the ratio, but restricting to [t0, t1] correctly attributes yield impacts to the period when the crop is actually present.

Model validation against the six-point shading dataset of  is reported in Section [sec:validation] (V1 validation, MAPE = 2.4% in the operational shading range Fshad≤0.25).

## Evapotranspiration Reduction and Water Savings

Reference evapotranspiration ET​0 is estimated hourly using the Hargreaves–Samani equation , driven by the daily temperature range and extraterrestrial radiation Ra computed astronomically from latitude and day of year following the standard FAO-56 formulation , requiring no ground radiation measurement. The Hargreaves–Samani method was selected over FAO-56 Penman-Monteith  because the PVGIS TMY files do not provide dew point temperature or net radiation as independent columns, making two of the four Penman-Monteith inputs unavailable without auxiliary decomposition. Hargreaves–Samani estimates of seasonal ET​0 agree with FAO-56 to within ±10% for the semi-arid and temperate climates represented in this study .

Under APV panels, evapotranspiration is reduced by the interception of incoming radiation. Following  and , the hourly ET under the APV array is:

ETAPV(t)=ET0(t)×1−αshade Fshad(t)

where αshade=0.40 is the ET reduction coefficient, empirically calibrated from field measurements of soil evaporation and canopy transpiration under partial shade . The value αshade=0.40 represents a conservative central estimate; field studies at GCR > 0.5 report values of 0.35–0.55 depending on crop canopy closure, soil type, and wind exposure. Sensitivity to αshade is discussed in Section [sec:discussion].

Annual growing-season water savings are:

$$\begin{equation}
        W_{\text{saved}} = \int_{t_0}^{t_1}
            \bigl[ET_0(t) - ET_{\text{APV}}(t)\bigr]\,dt
            = \alpha_{\text{shade}}\int_{t_0}^{t_1}
            F_{\text{shad}}(t)\,ET_0(t)\,dt
        \label{eq:wsaved}
    \end{equation}$$

The water savings Land Equivalent Ratio is defined as the fraction of growing-season ET​0 saved by the APV system:

LERwater=Wsavedt0t1ET0(t) dt×100%

Note on the denominator. Prior agrivoltaic studies have reported LERwater using annual ET​0 as the denominator , systematically underestimating the agronomic benefit by diluting growing-season savings with months when the crop is absent and the panels provide no agronomic water benefit. With a 122-day season (Almería, Ouagadougou), the annual denominator inflates by a factor of 365/122=3.0 relative to the growing-season denominator, reducing the reported LERwater by the same factor. The correction is largest at Almería and Ouagadougou (factor 3.0) and smallest at Konya (factor 1.9, 189-day season). Throughout this paper, LERwater uses the growing-season denominator (Equation [eq:lerwater]); the annual-denominator value is reported alongside for literature comparison only.

## Anaerobic Digestion Model

### Effective Digester Temperature

The effective digester operating temperature is determined by the ambient temperature profile and the thermal management strategy. PV electricity allocated to resistive heating (fPV,heat fraction of annual PV output) maintains the digester above a minimum operating threshold. The effective digester temperature is modelled as:

Tdig,eff=clip​Tamb,monthly+5, Tmin, Tref

where Tmin=26 °C is the minimum acceptable operating temperature below which methanogenesis becomes severely inhibited, Tref=37 °C is the mesophilic optimum, and the +5 ∘C boost above monthly mean ambient temperature represents the passive thermal gain from digester insulation and exothermic reaction heat . Monthly mean ambient temperatures from the PVGIS-SARAH3 TMY are used as the seasonal driver, reflecting the thermal inertia of a well-insulated CSTR digester that equilibrates on a monthly rather than hourly timescale.

### Arrhenius BMP Temperature Correction

The temperature-corrected biochemical methane potential follows an Arrhenius-type relationship :

BMP(Tdig,eff)=BMPref×exp​θ (Tdig,eff−Tref)

with reference BMP BMPref=300 NmL CH4 gVS−1 at Tref=37 °C and Arrhenius coefficient θ=0.069 °C​−1 . Annual methane production is:

VCH4=VStotal×BMP×ηcap

where BMP is the annual average BMP weighted by monthly VS throughput, ηcap=1.00 under baseline conditions (S0), and VStotal is from Equation [eq:vs]. Annual biogas energy is:

Ebiogas=VCH4×fCH4×LHVCH4

with methane fraction fCH4=0.60 and lower heating value LHVCH4=35.8 MJ Nm−3 . Validation of the BMP temperature model against the five-point experimental dataset of  is reported in Section [sec:validation] (V2 validation, MAPE = 26.9% in the operating range T≤37 °C). Sensitivity of biogas output and eLER to BMPref, capture efficiency ηcap, and volatile-solids fractions is assessed via Monte Carlo sampling over literature-reported ranges in Section [sec:discussion].

### Digester Heat Balance

The annual digester heat demand is the sum of conductive wall losses, substrate heating requirement, and the exothermic reaction credit :

Qheat=Qloss+Qinflow−Qreaction

Conductive wall losses from a cylindrical digester (height = diameter assumption) with effective heat transfer coefficient Ueff=0.8 W m−2 K−1 :

Qloss=Ueff AwallTdig−Tamb×87601000

where Awall is the total digester surface area (m​2) and the factor 8760/1000 converts from W to MWh yr​−1. Substrate heating demand:

Qinflow=VdigHRT×365×ρs×Cp×max​0, Tdig−Tin/3600

with substrate density ρs=1020 kg m−3, specific heat Cp=4.2 kJ kg−1 K−1, and inlet temperature Tin=15 °C. The exothermic reaction credit is estimated as 5% of gross biogas energy :

Qreaction=0.05×Ebiogas

### Energy Self-Sufficiency Ratio

The Energy Self-sufficiency Ratio quantifies the thermal margin of the integrated system:

ESR=EPV,total+EbiogasQheat

When ESR≫1, the combined PV and biogas output far exceeds the digester heating requirement, and zero biogas is consumed for self-heating. Under this condition, LERbiogas equals the ratio of APV-system biogas to open-field reference biogas by construction — a structural identity rather than an optimised outcome, as discussed in Section [sec:discussion].

## Extended Land Equivalent Ratio

The extended Land Equivalent Ratio (eLER) generalises the classical two-component agrivoltaic LER by adding net biogas energy as a third dimensionless land-use output:

eLER=Ycrop,AVYcrop,open⏟LERcrop+EPV,soldEPV,ref⏟LERPV+Ebiogas,APVEbiogas,ref⏟LERbiogas

An eLER greater than 1.0 indicates that the integrated APV–AD system uses land more productively than three separate monocultures — an open-field crop, a dense-pack ground-mount PV array, and a standalone AD unit — occupying the same total area. Equal weighting of the three components is adopted here, consistent with the original two-component LER framework, in which each term is already normalised to its own reference system and is therefore commensurable without further weighting; the sensitivity of eLER and cross-site rankings to alternative (energy-content- and revenue-weighted) schemes is examined in Section [sec:discussion]. Each component is defined as follows.

LERcrop is the yield ratio from Equation [eq:lercrop]: the ratio of growing-season PAR integral under the APV array to the open-field PAR integral, bounded in [0, 1] for tomato at the modelled irradiance levels.

LERPV is the ratio of annual APV electricity sold to grid to the reference PV output. Two reference definitions are used:

- Definition A (independent reference, primary): EPV,ref is computed from a simulated dense-pack ground-mount array at the same tilt angle β* with no inter-row shading (GCR = 1.0 in the limiting sense, i.e. rows spaced at exactly the module projected width Lmcosβ). This provides a non-circular reference that is independent of the APV design variables.

- Definition B (circular reference, for comparison): LERPV≡GCR, as the APV array output referenced to itself equals the fraction of ground covered. Reported for literature comparability with studies using this convention, but not used as the primary metric .

LERbiogas is the ratio of biogas energy produced by the APV–AD system to the output of a standalone reference AD unit. The reference AD unit processes the same land area under open-field conditions (LERcrop=1.0, maximum residue production) with identical digester volume, HRT, and manure co-substrate:

LERbiogas=Ebiogas,APVEbiogas,ref

Because the manure co-substrate (VSmanure) is identical in both the APV and reference cases, LERbiogas reflects exclusively the effect of APV shading on crop residue production. When LERcrop<1, the APV system produces fewer crop residues than the open-field reference, and consequently LERbiogas<1. This is a structural consequence of the shading-residue coupling, not a model artefact.

The two-component benchmark for literature comparison is:

LER2C=LERcrop+LERPV(Def. A)

The published two-component maximum is LER2C=1.94 from  for bifacial tomato in Arizona. All results in this study are reported under Definition A unless explicitly stated otherwise, and the numerical difference between Definition A and B is reported in Table [tab:eLER].

## Design Optimization

### Differential Evolution Algorithm

The six design variables and their bounds are summarised in Table 4. The objective function eLER(x) (Equation [eq:eler]) is maximised using Differential Evolution (DE) , a population-based global optimizer that requires no gradient information and handles non-smooth, multimodal objective landscapes efficiently. DE has been widely applied to renewable energy system sizing problems where the design space contains bound constraints and discrete-continuous interactions .

Design variable bounds for Differential Evolution. The inter-row clearance constraint (Equation [eq:overlap]) is enforced dynamically as a feasibility penalty during optimization.

| Variable | Symbol | Lower | Upper | Unit |
| --- | --- | --- | --- | --- |
| Panel tilt angle | β | 15 | 40 | ° |
| Row spacing | drow | 3 | 14 | m |
| Module bottom-edge height | Hm | 1.5 | 5.0 | m |
| Digester volume | Vdig | 2 | 50 | m³ |
| Hydraulic retention time | HRT | 20 | 40 | days |
| PV heating fraction | fPV,heat | 0.05 | 0.50 | — |

The DE configuration follows common recommendations :

- Population size: NP=15×nvar=90 individuals, initialised via Latin Hypercube Sampling with seed = 42 for reproducibility.

- Mutation factor: F dithered uniformly in [0.5, 1.0] at each generation, balancing global exploration with local convergence.

- Crossover probability: Cr=0.70 (binomial crossover).

- Maximum generations: Gmax=50.

- Local polishing: after DE convergence, the best candidate is refined using L-BFGS-B with bound constraints, recovering sub-pixel precision in the continuous design space.

Two feasibility constraints are enforced via an infeasibility penalty f=−∞ applied to violating candidates:

- Organic loading rate: OLR∈[1.5, 5.0] kg VS m−3d−1, ensuring stable digester operation without acidification or underloading.

- Inter-row clearance: drow≥Lmcosβ+0.5 m, preventing physical module overlap (Equation [eq:overlap]).

The complete pseudocode of the DE procedure is given in Appendix [app:de]. Each evaluation of the objective function eLER(x) runs the full six-model pipeline — solar geometry, PV model, crop yield, ET reduction, AD model, and eLER assembly — on the 8760-hour TMY dataset for the corresponding site. At NP=90 individuals and Gmax=50 generations, each site requires 90×50=4500 full pipeline evaluations, converging in 102–172 seconds on a standard desktop CPU.

### Boundary Convergence

At all four sites, the optimizer converges to the lower bounds of row spacing (drow*=3.03 m, constrained by the inter-row clearance condition) and module height (Hm*=1.50 m, the specified lower bound). This boundary convergence is a structural result: the eLER landscape reaches its optimum at the physical constraint boundary, where maximum panel density simultaneously maximises PV output and reduces the open-field PAR integral denominator (Equation [eq:lercrop]). The L-BFGS-B polishing step confirms that no interior local maximum exists in the neighbourhood of the boundary solution. The eLER penalty associated with any mechanical access requirement beyond the minimum clearance is quantified in the sensitivity analysis of Section [sec:discussion].

### Multi-Objective Extension (NSGA-II)

For the water trade-off analysis, the scalar eLER objective is replaced by a bi-objective problem: simultaneously maximise eLER and LERwater (Equation [eq:lerwater]). This is solved using NSGA-II  with 50 solutions and 80 generations implemented via the pymoo library . The bi-objective Pareto front provides the decision-maker with explicit trade-off information between land productivity and water conservation, enabling context-specific design choices for water-stressed smallholder systems.

## AD Valorization Scenarios

Seven economic scenarios (S0–S6) are evaluated to quantify the sensitivity of project economics to biogas valorization strategy and carbon market participation (Table 5). All scenarios share the same optimized APV–AD design (x*) and differ only in biogas price, BMP multiplier, capture efficiency, or additional revenue streams. eLER is invariant across scenarios by construction, since it depends solely on physical outputs (EPV,sold, Ycrop, Ebiogas) and not on market prices.

AD valorization scenarios S0–S6. All scenarios use the same optimized design x*. eLER is identical across scenarios; only revenue structure changes.

| Scenario | Description | Key modification |
| --- | --- | --- |
| S0 | Baseline: mono-digestion, 100% grid export | BMPmult=1.00, ηcap=1.00 |
| S1 | Co-digestion: +30% cattle manure | BMPmult=1.15 |
| S2 | Sub-optimal CH​4 capture (85%) | ηcap=0.85 |
| S3 | LPG displacement (on-site use) | pbg=$0.18 kWh−1 |
| S4 | Digestate N–P–K credit | N credit at $1.20 kg−1 N |
| S5 | Biomethane grid injection (EU premium) | pbg=5.5 pelec |
| S6 | Carbon credit (voluntary market) | pc = site-specific ($/tCO​2) |

Scenario S0 serves as the economic baseline: biogas is sold at the local grid biogas tariff pbg (Table 2) and all PV electricity is exported to grid at pelec after deducting the heating fraction fPV,heat.

Scenario S1 models co-digestion with a 30% increase in cattle manure beyond the baseline co-substrate, raising BMP by 15% through improved C:N ratio and microbial diversity . The additional manure input is assumed available at zero marginal cost from the same farm herd.

Scenario S2 models sub-optimal biogas capture at 85% efficiency, representing a system with gas leakage, flaring, or insufficient gas storage. It provides a lower bound on the biogas revenue stream.

Scenario S3 models on-site LPG displacement, where biogas is used directly for cooking or process heat at a price parity of $0.18 kWh​−1 — the average LPG equivalent price in the four study regions . This scenario is particularly relevant for Ouagadougou, where LPG import costs are high.

Scenario S4 adds a digestate nitrogen credit of $1.20 kg​−1 N, reflecting the avoided cost of equivalent synthetic urea fertilizer. Digestate N content is estimated as 0.003 kg N kg−1 fresh residue .

Scenario S5 models biomethane grid injection under European EEG 2023 and RED III incentives, where the effective biomethane market price is 5.5× the local electricity purchase price . This scenario is directly applicable to Almería and Freiburg; for Konya and Ouagadougou, which lack biomethane grid injection policy frameworks, S5 represents an aspirational upper bound pending regulatory development.

Scenario S6 adds a carbon credit revenue stream based on CO​2 displaced by PV electricity and biogas substituting fossil fuels. Carbon prices are site-specific: EU ETS market rate ($65 tCO​2−1) for Almería and Freiburg; voluntary Gold Standard rate ($12 tCO​2−1) for Ouagadougou; and Turkish voluntary carbon market rate ($15 tCO​2−1) for Konya. Annual CO​2 avoidance is:

CO2,avoided=EPV,sold×EFgrid+Ebiogas1000×EFgas

where EFgrid is the site-specific grid emission factor (Table 2) and EFgas=0.202 tCO2 MWh−1 is the natural gas emission factor from IPCC AR6 .

## 4E Analysis Framework

Investment-grade economic evaluation uses a 20-year project life with site-specific discount rates (Table 2), annualized CAPEX, and annual O&M fixed at 1.5% of total CAPEX. The 4E framework integrates four performance dimensions into a single comparative assessment across sites and scenarios.

### E1 — Energetic Analysis

The energetic dimension quantifies annual energy flows: PV total generation, PV sold to grid, biogas energy, digester heat demand, and ESR (Equation [eq:esr]). These are reported per hectare of land area for cross-site comparability.

### E2 — Economic Analysis

CAPEX is decomposed into four components:

- PV array (elevated APV structure): $0.63 W​p−1, reflecting the $0.45 W​p−1 utility-scale ground-mount benchmark  plus a 40% premium for elevated APV mounting structure, cable routing, and installation.

- AD digester: $800 m​−3 installed capacity , covering tank construction, mixing system, gas handling, and instrumentation for small farm-scale CSTR digesters (2–50 m​3).

- Agricultural infrastructure: $2 500 ha​−1 (FAO benchmark), covering drip irrigation installation, soil preparation, and access tracks.

- Balance of plant (BOP): 15% of (PV + AD) CAPEX, covering inverters, electrical wiring, civil works, and commissioning.

Annual gross revenue combines all income streams active under the relevant scenario:

Rgross=RPV+Rtomato+Rbiogas+Rdigestate+Rcarbon

Net annual cash flow, Net Present Value, Internal Rate of Return, and simple payback period are:

CFnet&=Rgross−CO&MNPV&=−CAPEX+CFnet×1−(1+r)−NrIRR&: 0=−CAPEX+CFnet×1−(1+IRR)−NIRRPB&=CAPEXCFnet

where r is the site-specific discount rate and N=20 years. IRR is solved by bisection on the NPV equation.

### E3 — Energo-Economic Analysis

The Levelised Cost of Electricity for the PV component isolates the cost of generating one kWh of PV electricity, accounting for both annualized PV CAPEX and the proportional share of O&M:

LCOEPV=CAPEXPV+CO&M,PV×1−(1+r)−NrEPV,sold×1−(1+r)−Nr

The levelised cost of biogas production is computed analogously using CAPEXAD and Ebiogas as the denominator, providing a cost-of-energy metric comparable to published small-scale AD benchmarks .

### E4 — Environmental Analysis

The environmental dimension quantifies annual CO​2 avoidance (Equation [eq:co2]) and its monetary value under site-specific carbon pricing (Table 2). Two displacement pathways are distinguished: grid electricity displaced by PV generation (valued at the grid emission factor EFgrid) and natural gas displaced by biogas (valued at the IPCC AR6 natural gas factor EFgas=0.202 tCO2 MWh−1 ). The carbon credit value Rcarbon=CO2,avoided×pc enters the revenue calculation of Scenario S6 (Equation [eq:revenue]). Table 6 summarises all fixed 4E analysis parameters.

Fixed parameters for the 4E economic and environmental analysis.

| Parameter | Symbol | Value | Source |
| --- | --- | --- | --- |
| Project lifetime | N | 20 yr | standard |
| O&M rate | fO&M | 1.5% CAPEX yr​−1 |  |
| PV CAPEX (APV) | cPV | $0.63 W​p−1 |  |
| AD CAPEX | cAD | $800 m​−3 |  |
| Agricultural CAPEX | cagri | $2 500 ha​−1 | FAO |
| BOP fraction | fBOP | 15% of (PV+AD) | standard |
| Natural gas EF | EFgas | 0.202 tCO​2 MWh​−1 |  |
| Digestate N content | — | 0.003 kg N kg​−1 |  |
| N credit price | pN | $1.20 kg​−1 N | market |
| LPG equivalent price | pLPG | $0.18 kWh​−1 |  |
| Biomethane premium (S5) | — | 5.5×pelec |  |