# Discussion

## The PAR-Saturation Mechanism: Implications for Site Selection and Crop Choice

The non-monotone relationship between annual solar resource and eLER — Freiburg (GHI = 1 150 kWh m​−2) outperforming Konya (GHI = 1 650 kWh m​−2) — is one of the key mechanistic findings of this study and requires careful interpretation before deployment implications can be drawn.

The mechanism operates through the PAR integral denominator in Equation [eq:lercrop]. In an open field, the denominator t0t1PARopen(t) dt accumulates all incoming PAR without a ceiling. At Konya, with a mean growing-season daytime GHI of approximately 450 W   m−2, the corresponding open-field PAR of 450×0.48=216 W m−2 exceeds PARsat=174 W m−2 for the majority of daylight hours. The denominator therefore grows faster than the numerator — which is clipped at PARsat in both the APV and the open field — making LERcrop structurally lower at high-irradiance sites regardless of the panel geometry.

At Freiburg, mean growing-season daytime GHI is approximately 320 W   m−2, giving open-field PAR of 154 W   m−2 — below PARsat. The denominator accumulates proportionally less uncapped PAR, and the ratio remains more favourable. This is not a statement about absolute crop yield: Freiburg produces less tomato per hectare in absolute terms (30 t ha​−1) than Konya (60 t ha​−1). It is a statement about the relative efficiency of land use: at Freiburg, the APV system captures a larger fraction of the agronomically useful PAR, because the open-field reference does not accumulate as much PAR above the saturation point.

Generalisation to other crops. The PAR-saturation mechanism is crop-specific. Tomato has a high light saturation point (PARsat=174 W m−2), making it sensitive to the mechanism at the irradiance levels of all four study sites. For shade-tolerant crops with lower saturation points — lettuce (76 W   m−2), spinach (80 W   m−2), or basil (100 W   m−2) — the denominator would be clipped at a much lower threshold, and open-field PAR would exceed PARsat for a larger fraction of daylight hours at all sites, potentially eliminating the eLER difference between high- and low-irradiance locations. For high-saturation crops such as maize (PARsat≈400 W m−2), the mechanism would be reversed: open-field PAR rarely reaches saturation, the denominator grows proportionally with irradiance, and high-irradiance sites would achieve higher LERcrop, restoring a monotone eLER–GHI relationship. Multi-crop generalisation is therefore identified as an important extension of the present framework.

Site selection implication. The conventional wisdom in agrivoltaic deployment — that high-irradiance semi-arid sites are preferable because they maximise PV revenue — requires qualification when high-PAR crops such as tomato are cultivated. The present results suggest that moderate-irradiance temperate sites offer a comparable or superior eLER for tomato, because the PAR available in those climates is more fully utilisable by the crop. From a food-energy-water nexus perspective, the optimal site is not the site with the most sun, but the site where the solar resource best matches the crop’s photosynthetic capacity. This reframes the site selection problem from a single-objective (maximise PV output) to a multi-objective (maximise eLER) decision, consistent with the optimization framework developed in this study.

## Boundary Convergence: Physical Meaning and the Access Trade-off

The convergence of the optimizer to the lower bounds of both row spacing (drow*=3.03 m) and module height (Hm*=1.50 m) at all four sites is a structural result that reveals the shape of the eLER landscape and has direct practical implications for APV system design and agricultural operations.

Why the optimizer converges to the boundary. Increasing panel density — reducing drow and Hm toward their lower bounds — has two simultaneous effects on eLER. First, it increases LERPV: more modules per hectare generates more electricity from the same land area, raising the numerator EPV,sold relative to the reference. Second, it increases Fshad, which reduces PARopen denominator in Equation [eq:lercrop] by intercepting more incident radiation. Both effects push eLER upward. The only countervailing force is the reduction in LERcrop: more shading reduces PAR available to the crop, lowering the numerator ∫min(PARAV,PARsat) dt. The optimizer converges to the physical constraint boundary because, at the parameter values of this study (tomato, GHI = 1 150–2 050 kWh m​−2), the PV density gain and denominator reduction outweigh the crop yield penalty up to the point where physical overlap between rows prevents further densification.

The eLER penalty of mechanical access. In practice, agricultural machinery requires a minimum inter-row clearance for tractors, harvesters, and irrigation equipment. Typical tractor widths of 1.8–2.2 m, combined with operational clearance, require row spacings of 3.5–5.0 m in mechanised farming systems. The eLER cost of this access requirement can be quantified from the boundary convergence result: since the optimizer seeks the minimum feasible drow at all sites, each additional metre of required row spacing represents a deviation from the eLER optimum. From the sensitivity of eLER to drow in the neighbourhood of the boundary solution, the eLER penalty is approximately −0.04 to −0.06 per additional metre of row spacing, with the exact value depending on site irradiance and tilt angle. [This sensitivity figure was not re-verified against the corrected model in the present revision and should be re-computed from a drow sweep before the manuscript is finalised; it was not affected by any of the six corrections described in this section but was also not directly re-derived from Table [tab:optimal]’s updated design point.] A mechanised farming system requiring drow=5.0 m instead of 3.03 m would sacrifice approximately 0.08–0.12 eLER units — a penalty of 5–7% relative to the boundary optimum.

Practical design recommendations. Three design strategies can partially recover the access penalty while maintaining agricultural operability:

- Smallholder manual cultivation. For systems serving smallholder farmers who operate without mechanised equipment — the predominant farming model in Burkina Faso and much of the semi-arid developing world — the minimum physical clearance of 3.03 m is operationally feasible. Manual cultivation, hand harvesting, and drip irrigation require only 0.8–1.2 m of working clearance, making the boundary-convergence design directly deployable at Ouagadougou-type sites.

- Increased module height. Raising Hm from 1.50 m to 2.5 m allows agricultural machinery to pass beneath the panels even at reduced row spacing, recovering mechanical access without increasing drow. The eLER cost of this height increase is modest: Hm affects Fshad only through the shadow length formula (Equation [eq:shading]), and at drow=3.03 m the shading fraction is already near its maximum for the relevant solar elevation angles.

- Shade-adapted crop selection. Choosing crops with lower PARsat (lettuce, spinach, herbs) reduces the eLER penalty of increased row spacing because the crop is less sensitive to the absolute PAR reduction under denser panel configurations. For such crops, a wider drow may achieve a similar or higher LERcrop as the boundary-convergence design for tomato.

Comparison with prior literature. The boundary convergence on row spacing is consistent with the findings of , who reported that LER is monotonically increasing with GCR for lettuce up to GCR = 0.65 before levelling off, and with , who found that the LPF-optimal design for bifacial tomato in Arizona converged to the maximum permissible GCR under their access constraints. The present study extends these findings by providing an explicit quantification of the access penalty and by demonstrating that the boundary convergence is climate-independent — it occurs at all four sites despite GHI varying by a factor of 1.8 and mean annual temperature varying by 17 °C.

## The eLER Metric: Interpretation, Definitions, and Structural Properties

Four structural properties of the eLER metric require explicit interpretation to avoid misreading the numerical results reported in Section [sec:results_eler].

Property 1 — LERbiogas<1 is a coupling consequence, not a system failure. The result LERbiogas=0.611–0.729 at all sites means that the APV–AD system produces less biogas than a standalone reference AD unit processing the same land area under open-field conditions. This is not a design failure or an inefficiency of the integrated system: it is a direct and unavoidable consequence of the shading-residue coupling. When panels shade the crop, LERcrop<1, meaning fewer crop residues are harvested per hectare. Fewer residues reduce VS input to the digester, which reduces biogas output. The reference AD unit, by contrast, operates with open-field residues at LERcrop=1.0 — maximum residue production. The ratio of the two biogas outputs is therefore structurally less than 1.0 whenever shading is present.

This coupling has an important implication for the interpretation of the eLER sum. The three eLER components are not independent: LERcrop and LERbiogas are negatively correlated through the shading-residue pathway. Increasing GCR raises LERPV and reduces LERcrop, which in turn reduces LERbiogas. The optimizer finds the design that maximises the sum of all three components, accepting the biogas penalty in exchange for the larger PV gain.

Property 2 — An alternative LERbiogas definition changes the interpretation but not the system physics. An alternative reference for LERbiogas could use a standalone AD unit operating without PV heating — at ambient temperature rather than at the PV-assisted Tdig,eff. Under this definition:

LERbiogas,alt=Ebiogas,APVEbiogas,ambient

where Ebiogas,ambient is computed using Tdig,eff=Tamb without the 5 °C boost or the [26, 37] °C clip. Under this definition, LERbiogas,alt>1 at Konya and Freiburg in winter months, explicitly quantifying the PV thermal subsidy to AD performance. This alternative is conceptually more informative about the PV–AD synergy but harder to compare across studies because the reference changes with ambient temperature. The primary definition used throughout this paper (Equation [eq:lerbiogas]) was chosen for consistency with the classical LER framework, in which all references are activity-specific open-field monocultures operating under the same boundary conditions as the integrated system.

Property 3 — ESR ≫ 1 is a design identity, not a performance finding. ESR values of 453–2 000 at the optimised designs reflect the fundamental size mismatch between the PV array (which generates electricity at the scale of MWh ha​−1 yr​−1) and the small farm-scale digester (which demands heat at the scale of MWh ha​−1 yr​−1 — the same unit but three orders of magnitude smaller in absolute value). With Vdig*=2.64–4.32 m​3 and heat demand of only 0.72–2.26 MWh ha​−1, even 5% of PV output provides 50–72 MWh of thermal energy — 23–100 times the annual heat demand. The ESR is therefore more a confirmation of the PV array being appropriately scaled to the land area than a meaningful performance indicator for the AD system. Future studies operating at larger scale (community digesters serving multiple hectares) would achieve lower ESR values that are more informative about the thermal balance between PV and AD.

Property 4 — Equal component weighting is the methodologically consistent choice, not an unexamined default. Equation [eq:eler] sums LERcrop, LERPV, and LERbiogas with implicit equal weight. Because each component is already a ratio to its own reference system — the same normalisation the classical two-component LER uses to make physically incommensurable outputs (kg of crop, kWh of electricity, m​3 of biogas) comparable as land-equivalent fractions — equal weighting is not an unexamined default but the choice consistent with that normalisation logic. Re-weighting the three components by their raw physical magnitude would reintroduce exactly the scale problem the ratio normalisation is designed to solve, since PV output exceeds crop and biogas output by 50–100× in raw energy or monetary terms at every site studied here (e.g. at Freiburg, PV output is 938 MWh   ha−1 versus a crop energy-content equivalent of only 9.6 MWh   ha−1).

This claim was tested directly rather than asserted. Two magnitude-based alternative weighting schemes were constructed — an economic scheme (weights proportional to each component’s share of Scenario S0 gross revenue) and an energy-content scheme (weights proportional to each component’s physical energy content, converting crop yield to its caloric equivalent) — and applied to the optimised design at each site. Table 1 summarises the effect on the cross-site eLER ranking.

Sensitivity of the eLER site ranking to component weighting scheme, evaluated at each site’s optimised design (Table [tab:optimal]). Equal weighting is the primary metric used throughout this paper (Table [tab:eLER]).

| Weighting scheme | Site ranking (highest eLER first) |
| --- | --- |
| Equal (primary metric) | Freiburg > Ouagadougou > Almería > Konya |
| Energy-content | Ouagadougou > Almería > Konya > Freiburg |
| Economic (revenue-share) | Ouagadougou > Almería > Freiburg > Konya |

Both magnitude-based schemes assign PV a normalised weight of approximately 2.7–3.0 out of 3 at every site — effectively collapsing eLER to a PV-only metric — and both displace Freiburg from the top rank, with the energy-content scheme placing it last despite its highest equal-weighted eLER (Table [tab:eLER]). This confirms the ranking sensitivity is not merely hypothetical. However, the design recommendations of this study are materially more robust than the ranking: re-optimising the full six-variable Differential Evolution problem under economic weighting reproduces the same boundary convergence on row spacing and module height (Section [sec:results_optimal]) at every site, with optimal tilt angles shifting by less than 1​∘ from the equal-weighted solution. Because PV revenue dominates under any economically-motivated weighting, “maximise PV density” remains close to optimal regardless of the exact weight assigned to the smaller crop and biogas terms.

The appropriate reading of this result is not that eLER’s site ranking is fragile in a way that undermines the metric, but that ranking sites by a magnitude-weighted composite is a poor fit for a LER-family metric specifically: doing so discards the very information the ratio normalisation was constructed to preserve. eLER, like the classical two-component LER before it, is a decision-support metric reflecting an explicit modelling choice about how to compare heterogeneous land-use outputs — not a universal welfare measure with a single objectively correct weighting. Equal weighting is defended here as the choice consistent with the metric’s own normalisation logic and is retained as the primary metric on that basis, but a different decision context (for example, a food-security policy analysis that deliberately up-weights LERcrop, or an investor analysis that deliberately weights by revenue) could justify a different, explicitly-stated weighting for its own purposes. The PAR-saturation non-monotonicity (Finding C2) and the boundary-convergence result (Finding C3) are reported under equal weighting as the primary metric, with the ranking’s sensitivity to alternative schemes disclosed here as a scope limitation of that choice rather than treated as a threat to the paper’s structural conclusions. Full weighting derivations and additional detail are provided in the supplementary sensitivity analysis (docs/eLER_weighting_sensitivity.md).

Beyond the scalar optimum: a Pareto perspective. Property 4 frames eLER as one explicit, defensible decision rule among several; Section [sec:optimizer] (Multi-Objective Extension) noted that the scalar objective can be replaced by a bi-objective problem maximising eLER and LERwater jointly via NSGA-II. Running this bi-objective search for Konya (50 solutions, 80 generations, pymoo) makes the trade-off concrete rather than abstract. Figure 1 shows the resulting Pareto front: eLER and LERwater trade off nearly linearly across the feasible design space, from (eLER =1.20, LERwater=36.1%) at the water-maximising end to (eLER =1.64, LERwater=18.9%) at the eLER-maximising end. The single-objective optimum reported in Table [tab:eLER] sits exactly at this second extreme — confirming the scalar optimizer is not leaving an easy joint improvement on the table, but also making explicit that it is one specific point on a continuum, not the only defensible design. A water-stressed smallholder context (Section [sec:results_water]) might reasonably trade roughly 0.1 eLER units for several percentage points of additional LERwater; a context prioritising maximum land-use productivity would not. This is precisely the kind of design choice a decision-support metric should expose rather than resolve internally, reinforcing Property 4’s framing.

Pareto front between eLER (Definition A) and LERwater (growing-season, Equation [eq:lerwater]) for Konya, from a bi-objective NSGA-II search (50 solutions, 80 generations) over the same six-dimensional design space used for the primary scalar optimization. The dotted line marks the scalar eLER optimum reported in Table [tab:eLER], which coincides with the eLER-maximising end of the front.

Definition A versus Definition B: when to use each. Definition A (LERPV referenced to a dense-pack independent array) is the scientifically rigorous choice for studies that aim to quantify the net land-use benefit of the APV configuration relative to what could be achieved with two separate monocultures. Definition B (LERPV≡GCR) is appropriate only for literature comparison with studies that have used the circular reference, and its numerical values should always be flagged as such. The difference between the two definitions is 0.029–0.033 in LERPV and 0.029–0.033 in eLER at the optimised designs of this study (Table [tab:eLER]), which is small relative to the between-site eLER variation (0.123) but non-negligible relative to the between-site LERcrop variation (0.041). Standardisation of the PV reference definition — ideally through an agreed protocol analogous to the IEC 61724 standard for PV performance reporting — is identified as a priority for the agrivoltaic community.

## Economic Viability and Policy Implications

The 4E analysis reveals a wide range of economic performance across the four sites, from strongly viable (Ouagadougou, IRR = 28.1%) to marginal but positive (Konya, IRR = 11.0% against an 8% discount rate). Three policy-relevant implications emerge from the economic results.

Electricity tariff structure is the primary economic determinant. The largest revenue stream at all four sites is PV electricity sales, which accounts for 92–96% of gross revenue under Scenario S0. The electricity price therefore has a disproportionate influence on project economics relative to tomato price, biogas price, or carbon credit. At Konya, the low Turkish electricity tariff ($0.06 kWh​−1) is the binding constraint on IRR: with a broadly comparable PV output to Almería (1 295 vs. 1 386 MWh ha​−1 yr​−1 at optimal design), Konya generates disproportionately less PV revenue simply because the tariff is 33% lower ($0.06 vs. $0.09 kWh​−1). A sensitivity analysis shows that a Turkish electricity price increase to $0.09 kWh​−1 — consistent with the regional trend as fossil fuel subsidies are progressively reduced — would raise Konya’s IRR to approximately 18%, exceeding Freiburg’s current IRR (13.0%) and well above the discount rate. This underscores that APV–AD investment viability in semi-arid developing economies is primarily a function of energy price reform, not of solar resource or technology cost.

The carbon credit scenario (S6) provides the strongest economic uplift in this study and is worth prioritising in policy design. S6 raises IRR by 1.7–4.6 percentage points above S0 at all sites, with the largest absolute improvement at Freiburg (+4.6 pp) driven by the EU ETS price ($65 tCO​2−1). At Ouagadougou and Konya, where voluntary carbon market prices are much lower ($12–15 tCO​2−1), the S6 improvement is smaller in absolute terms (+1.9 and +1.7 pp respectively) but still meaningful relative to the discount rate. The policy implication is that extending carbon credit mechanisms — whether through the Article 6 Paris Agreement framework, Gold Standard certification, or national carbon pricing — to smallholder agrivoltaic installations in Sub-Saharan Africa and Central Asia would provide a material improvement in project economics without requiring technology subsidies or feed-in tariff reform. Given that the 861 tCO​2 ha​−1 yr​−1 avoided at Ouagadougou represents a significant fraction of the annual per capita CO​2 budget compatible with 1.5 °C pathways, this mechanism also aligns economic incentives with climate goals.

Ouagadougou provides the strongest investment case among the four sites in this study. An IRR of 28.1% under S0 and 29.9% under S6, combined with a payback of 3.3–3.5 yr, makes the Ouagadougou configuration attractive even at the 10% discount rate that reflects the higher cost of capital in developing economies. Three structural advantages drive this result: the highest electricity price in the study ($0.12 kWh​−1, reflecting the true cost of diesel generation), the highest biogas yield (5.98 MWh ha​−1, from the optimal Arrhenius BMP at 33.2 °C average digester temperature), and the largest CO​2 avoidance (861 tCO​2 ha​−1, from the high grid emission factor of 0.632 tCO​2 MWh​−1). The small digester volume (2.64 m​3) and low AD CAPEX ($2 112) mean that the biogas revenue stream is essentially free cash flow on top of the PV investment. For development finance institutions seeking bankable climate-positive agricultural projects in the Sahel, the APV–AD model at Ouagadougou-type sites offers a notable combination of high IRR, short payback, large CO​2 impact, and food security co-benefit.

LCOE is at or below utility-scale solar at every site, not just the best ones. LCOE​PV ranges from $0.038 kWh​−1 at Almería to $0.049 kWh​−1 at Freiburg, against the IRENA 2023 global utility-scale solar benchmark of $0.049 kWh​−1. Almería is 22% below the benchmark; Konya ($0.047 kWh​−1) and Ouagadougou ($0.048 kWh​−1) are 2–4% below it; Freiburg is essentially at parity. No site exceeds it, despite the elevated APV structure ($0.63 W​p−1 vs. $0.45 W​p−1 for ground-mount ) carrying a 40% CAPEX premium and the inter-row shading loss at GCR = 69–72% reducing annual specific yield below what a dense-pack unshaded array would achieve. This is a stronger result than the PV-only LCOE metric itself suggests: it is a single-output comparison that ignores crop production, water savings, and biogas, none of which are credited here. A full value-stack LCOE that credits all outputs would likely be lower than the PV-only LCOE at every site, and because the PV-only LCOE already clears the utility-scale benchmark everywhere, the full value-stack LCOE would likely remain below that benchmark at all four sites.

## Limitations and Future Work

Six limitations of the present modelling approach are identified, each motivating specific future research directions. Table 2 summarises them for quick reference; full discussion of each follows.

Summary of model limitations. “Affects eLER?” indicates whether the limitation propagates to the headline eLER metric (Table [tab:eLER]) or is confined to a secondary output.

| ID | Limitation | Quantified impact | Affects eLER? | Mitigation status |
| --- | --- | --- | --- | --- |
| L1 | Single-exponential Arrhenius BMP model underestimates methane yield at 20–30 °C | −30 to −52% BMP bias; ±43–45% 90% CI from parameter uncertainty | No (±2–2.4%) | Documented; Monte Carlo sensitivity run (docs/ad_bmp_sensitivity.md) |
| L2 | Single-crop scope (tomato only); PAR-saturation mechanism is crop-specific | Direction/magnitude of Finding C2 may differ for other crops | Yes (structural) | Not yet run; requires only a PARsat sweep |
| L3 | Hargreaves–Samani ET​0 omits humidity/wind; fixed αshade | ±10% vs. FAO-56 Penman-Monteith; ±25% via αshade range | No | Documented (docs/et0_ra_fix_notes.md); Penman-Monteith upgrade not yet implemented |
| L4 | No digestate-to-crop nutrient feedback modelled | Digestate N credit (S4) only; yield feedback (10–25%) not modelled | Yes (would raise it) | Deferred (Gap G6); requires multi-season field data |
| L5 | Static economic parameters (no price trajectories) | Konya’s S0 IRR margin is narrow (3.0 pp) and price-sensitive | No (economics only) | Not yet run; Monte Carlo price model proposed |
| L6 | Single-hectare normalisation; ESR (453–2 000) reflects smallholder scale | Larger-scale CAPEX/ESR would differ non-linearly | No (scale only) | Not yet run; 1–50 ha scale-up proposed |

L1 — BMP temperature model accuracy (V2 MAPE = 26.9%). The single-exponential Arrhenius model systematically underestimates BMP by 30–52% at temperatures of 20–30 °C, the range relevant to cold-season operation at Konya and Freiburg. The root cause is that a single Arrhenius coefficient calibrated at 37 °C cannot capture the inflected microbial response curve in the mesophilic lower range, where community adaptation provides higher-than-exponential yields . Future work should implement a piecewise Arrhenius model with separate coefficients above and below 30 °C, or a modified Gompertz kinetic model, calibrated against the multi-temperature BMP datasets now available for tomato residues and cattle manure . Until such a model is validated, all biogas yield and economic results for cold-climate sites should be treated as conservative lower bounds, with the true values likely 15–30% higher. A Monte Carlo sensitivity analysis over literature-reported ranges for BMPref, capture efficiency, and volatile-solids fractions (500 draws per site) shows this parameter uncertainty propagates to a ±43–45% 90% confidence interval on absolute biogas energy, while eLER itself remains within ±2–2.4% — robust because LERbiogas is a ratio referenced against a standalone AD unit using the same sampled parameters, which largely cancels the shared uncertainty (docs/ad_bmp_sensitivity.md).

L2 — Single-crop scope. All results are conditioned on tomato (Solanum lycopersicum) with PARsat=174 W m−2. The PAR-saturation mechanism that drives the non-monotone eLER–GHI gradient is crop-specific, and the magnitude and direction of this effect will differ substantially for other crops. Shade-tolerant crops with lower PARsat (lettuce at 76 W   m−2; spinach at 80 W   m−2) would likely eliminate or reverse the Freiburg–Konya eLER inversion, while high-saturation crops such as maize (400 W   m−2) would restore a monotone eLER–GHI relationship. A multi-crop sensitivity analysis — requiring only a change of the PARsat parameter in the existing model — is an important next extension, as it would help determine how broadly the PAR-saturation finding generalises beyond tomato.

L3 — Evapotranspiration model simplification. Reference ET​0 is estimated using the Hargreaves–Samani equation with extraterrestrial radiation computed astronomically (FAO-56), which agrees with FAO-56 Penman-Monteith to within ±10% in semi-arid and temperate climates  but cannot capture humidity and wind effects on ET under the APV canopy. Full derivation of the ET​0 formulation, including two corrections made during model development, is provided in docs/et0_ra_ fix_notes.md. The ET reduction coefficient αshade=0.40 is taken from field measurements under similar APV configurations  but may vary by ±0.10–0.15 depending on crop canopy closure, soil type, and wind exposure. A ±25% variation in αshade propagates to a ±25% variation in Wsaved and LERwater, which does not affect eLER or economics but is relevant to water-stressed deployment contexts. Future work should implement FAO-56 Penman-Monteith with auxiliary humidity and aerodynamic resistance terms derived from the PVGIS wind speed column, and validate αshade against lysimeter measurements at each study site.

L4 — Absence of digestate-to-crop nutrient feedback (G6). The model treats the AD digester as a terminal sink for crop residues: digestate is valued only as a tradeable nitrogen credit (Scenario S4) without modelling its effect on soil nutrient availability and subsequent crop yield. In practice, returning digestate to the APV field closes a nutrient loop that can increase fruit yield by 10–25% relative to unfertilised conditions , which would raise both the crop revenue and the VS input to the digester in subsequent seasons. Modelling this feedback requires multi-season soil nutrient dynamics that are beyond the scope of the present single-year TMY-based framework. It is identified as Gap G6 and an important direction for experimental field validation, requiring co-located APV–AD installations with controlled digestate application trials over at least three growing seasons.

L5 — Static economic parameters. All economic results use fixed prices for electricity, tomato, biogas, and carbon credits, without modelling price trajectories over the 20-year project life. PV module costs have declined at approximately 10–15% per year over the past decade , and biomethane prices are subject to regulatory risk in all four jurisdictions. A stochastic techno-economic model incorporating Monte Carlo sampling of price distributions — including correlated electricity and carbon price scenarios consistent with IEA Net Zero pathways  — would provide more realistic NPV confidence intervals and identify the price combinations under which each site crosses the investment threshold. Such a model would be particularly valuable for Konya, where the S0 IRR margin above the discount rate is narrow (3.0 pp) and highly sensitive to electricity price assumptions.

L6 — Single-hectare normalisation and scalability. All results are normalised to 1 ha, which is appropriate for smallholder farming contexts (Ouagadougou, Konya) but may not represent the economics of larger commercial APV–AD installations. At larger scale (10–100 ha), several cost components change non-linearly: AD digester CAPEX per m​3 decreases with scale (economies of scale in tank construction and gas handling), BOP costs per kWp decrease with larger inverter units, and the ESR — currently 453–2 000 — would decrease toward more physically meaningful values as the digester is sized to process residues from multiple hectares. A scale-up analysis from smallholder (1 ha) to commercial (50 ha) would provide the full range of deployment contexts and is a natural next step following experimental validation of the single-hectare model.