# : Differential Evolution Algorithm — Complete Pseudocode

Algorithm [alg:de] provides the complete pseudocode of the Differential Evolution procedure used to maximise the eLER objective function (Equation [eq:eler]) across the six-dimensional APV–AD design space. The algorithm is implemented in Python using scipy.optimize.differential_evolution with the settings described in Section [sec:optimizer], followed by scipy.optimize.minimize with the L-BFGS-B method for local polishing.

Initialisation

Main loop

Local polishing (L-BFGS-B) x*←argmaxi fi(Gmax) x*←L-BFGS-B​x*, lb, ub eLER*←eLER(x*)

x*, eLER*

# : Model Parameters — Complete Reference Table

Table 1 consolidates all fixed model parameters used throughout the study for reproducibility.

Complete list of fixed model parameters.

| Parameter | Symbol | Value | Unit | Source |
| --- | --- | --- | --- | --- |
| PV module (Jinko Tiger Neo 550 W​p bifacial) |  |  |  |  |
| Rated power (STC) | PSTC | 550 | W​p |  |
| Module length | Lm | 2.278 | m |  |
| Module width | Wm | 1.134 | m |  |
| Bifaciality factor | ϕ | 0.70 | — |  |
| Temp. coefficient | βp | −0.0035 | K​−1 |  |
| System losses | ηloss | 0.12 | — |  |
| PV thermal model (Faiman 2008) |  |  |  |  |
| Free convection coeff. | U0 | 25 | W m​−2 K​−1 |  |
| Wind convection coeff. | U1 | 6.84 | W s m​−3 K​−1 |  |
| Ground albedo | ρg | 0.25 | — |  |
| Rear irradiance fraction | frear | 0.12 | — |  |
| Crop yield model |  |  |  |  |
| PAR fraction of GHI | fPAR | 0.48 | — |  |
| Tomato PAR saturation | PARsat | 174 | W m​−2 |  |
| ET reduction model |  |  |  |  |
| ET shading coefficient | αshade | 0.40 | — |  |
| Anaerobic digestion model |  |  |  |  |
| Reference BMP | BMPref | 300 | NmL CH​4 gVS​−1 |  |
| Reference temperature | Tref | 37 | °C |  |
| Arrhenius coefficient | θ | 0.069 | °C−1 |  |
| Min. operating temp. | Tmin | 26 | °C |  |
| Temp. boost above ambient | — | 5 | °C |  |
| Methane fraction | fCH4 | 0.60 | vol/vol |  |
| Methane LHV | LHVCH4 | 35.8 | MJ Nm​−3 |  |
| Wall heat transfer coeff. | Ueff | 0.8 | W m​−2 K​−1 |  |
| Substrate density | ρs | 1 020 | kg m​−3 | standard |
| Substrate specific heat | Cp | 4.2 | kJ kg​−1 K​−1 | standard |
| Inlet temperature | Tin | 15 | °C | assumed |
| Reaction heat fraction | — | 0.05 | — |  |
| Tomato residue DM | DM | 0.06 | kg kg​−1 |  |
| Residue VS fraction | VSf | 0.85 | kg kg​−1 |  |
| Manure DM | DMm | 0.20 | kg kg​−1 |  |
| Manure VS fraction | VSm | 0.80 | kg kg​−1 |  |
| Residue-to-fruit ratio | Rres | 0.80 | kg kg​−1 |  |
| Economic analysis |  |  |  |  |
| Project lifetime | N | 20 | yr | standard |
| O&M rate | fO&M | 1.5 | % CAPEX yr​−1 |  |
| PV CAPEX (APV) | cPV | 0.63 | $ W​p−1 |  |
| AD CAPEX | cAD | 800 | $ m​−3 |  |
| Agricultural CAPEX | cagri | 2 500 | $ ha​−1 | FAO |
| BOP fraction | fBOP | 15 | % of (PV+AD) | standard |
| Natural gas EF | EFgas | 0.202 | tCO​2 MWh​−1 |  |
| Digestate N content | — | 0.003 | kg N kg​−1 |  |
| N credit price | pN | 1.20 | $ kg​−1 N | market |
| LPG equivalent price | pLPG | 0.18 | $ kWh​−1 |  |
| Biomethane premium (S5) | — | 5.5×pelec | — |  |