ERCİYES ÜNİVERSİTESİ FEN BİLİMLERİ ENSTİTÜSÜ # TEZ ÖNERİSİ FORMU # (İLK ÖNERİ)

24/08/2026 FEN BİLİMLERİ ENSTİTÜSÜ MÜDÜRLÜĞÜNE 4012540005 nolu Enerji Sistemleri Mühendisliği Anabilim Dalı yüksek lisans Türkçe/İngilizce öğrencisi Amidou MAIGA derslerini tamamlamış ve tez bilgi formunu ilk kez hazırlamıştır. İlgili tez bilgi formu ve formun değerlendirilmesine ilişkin Ana Bilim Dalı Kurulu kararı ekte sunulmuştur. Bilgilerinize arz ederim. Prof. Dr. Gamze GENÇ (Title, Name Surname Signature) Anabilim Dalı Başkanı (Head of Department) EKLER: 1- Ana Bilim Dalı Kurul Kararı 2- Transkript 3- YÖK Tez Veri Giriş Formu 4- Etik Kurul Kararı (Bu çalışma için Etik Kurul iznine gerek vardır işaretlendi ise) 5- İş Sağlığı ve Güvenliği Eğitimini aldığını belgelendirmesi. 6-Temel Laboratuvar Güvenliği Eğitimini aldığını belgelendirmesi. ÖNEMLİ NOT: Almış Oldukları Kredili Dersler ve Seminer dersinden başarısız olan öğrenciler Tez Dönemine Geçemeyeceklerinden Tez Bilgi Formunu Doldurmalarına Gerek Yoktur.

ANA BİLİM DALI AKADEMİK KURUL KARARI; Yıl; Toplantı No.; Karar No.; Tarih. 2026; 24/08/2026; **; 24/08/2026. GÜNDEM: Amidou MAIGA’ nın Tez Bilgi Formunun görüşülmesi. KARAR: Anabilim dalımız yüksek lisans öğrencisi Amidou MAIGA’ nın tez önerisi formu, Erciyes Üniversitesi Lisansüstü Eğitim ve Öğretim Yönetmeliğinin 40. maddesi gereği incelenerek tez başlığı ve içeriği itibariyle uygun olduğuna ve konunun Enstitü Müdürlüğüne sunulmasına oy birliği / oy çokluğu ile karar verilmiştir. Anabilim Dalı Akademik Kurulu (Department Academic Committee). Unvanı, Adı ve Soyadı (Title, Name and Surname); İmzası (Signature). Anabilim Dalı Başkanı (Head of Department); Prof. Dr. Gamze GENÇ. Üye (Member); Prof. Dr. M. Serdar GENÇ. Üye (Member); Prof. Dr. Nesrin KAYATAŞ DEMİR. Üye (Member); Doç. Dr. Gülşah ELDEN. Üye (Member); Doç. Dr. M. Fatih KAYA. Üye (Member); Doç. Dr. S. Buğra SELÇUKLU. Üye (Member); Doç. Dr. H. Hakan AÇIKEL.

| ERCİYES ÜNİVERSİTESİ FEN BİLİMLERİ ENSTİTÜSÜ # TEZ ÖNERİSİ FORMU # (İLK ÖNERİ) |  |
| --- | --- |
| Öğrenci Bilgileri (Student Information) |  |
| Ad Soyadı: (Name Surname) | Amidou MAIGA |
| Öğrenci Numarası: (Student ID Number) | 4012540005 |
| Anabilim Dalı: (Department) | Enerji Sistemleri Mühendisliği / Energy Systems Engineering |
| Programı: (Program) | Yüksek Lisans (Tezli) — İngilizce / MSc Thesis Program — English |
| E-Posta : (E-Mail) | 4012540005@erciyes.edu.tr |
| Telefon : (Phone) | 0536 717 0586 |
| Adres: (Address) | 38030, Köşk, Melikgazi/Kayseri - Turkey |
| Tez Danışmanı: (Thesis Advisor) | Prof. Dr. Gamze GENÇ (Title, Name Surname Signature) Anabilim Dalı Başkanı (Head of Department) |
| Tezin Adı: (Thesis Title) Integrated Bifacial Agrivoltaic–Anaerobic Digestion Optimization Across Four Climate Zones: A Novel Extended Land Equivalent Ratio Framework for the Food–Energy–Water Nexus |  |
| Tezin Özeti: (Thesis Abstract) Land-use competition between food production and renewable energy deployment is a major challenge in semi-arid regions. This thesis develops a coupled six-model optimization framework for bifacial agrivoltaic (APV) systems integrated with on-farm anaerobic digestion (AD) across four climate zones, Konya (BSk, Turkey), Almería (BSh, Spain), Ouagadougou (BSh, Burkina Faso), and Freiburg (Cfb, Germany), using hourly PVGIS-SARAH3 climate data. Six design variables (panel tilt, row spacing, module height, digester volume, hydraulic retention time, and PV heating fraction) are jointly optimized using Differential Evolution to maximize a novel three-component extended Land Equivalent Ratio (eLER), defined as the sum of crop, photovoltaic, and biogas land-equivalent terms. Optimized eLER values range from 1.641 to 1.764 across the four sites, while the classical two-component agrivoltaic LER alone remains above unity (1.030–1.088), confirming that the integrated system outperforms separate monocultures even before biogas valorisation. A counter-intuitive result emerges: Freiburg, with the lowest annual solar resource, achieves the highest eLER, driven by a PAR-saturation mechanism at higher-irradiance sites. A techno-economic (4E: Energetic, Economic, Energo-economic, Environmental) analysis shows PV yields of 1,002–1,433 MWh ha⁻¹, net present values of $134k–$802k, and internal rates of return of 11.0%–28.1% under baseline conditions, rising to 12.7%–29.9% under a carbon-credit scenario. Growing-season water savings of 81–159 mm correspond to 15.6%–22.4% of seasonal evapotranspiration demand. The framework is validated at the component level against independent literature datasets (crop yield, biogas temperature response) and, externally, against the pvlib solar-modelling library; a Monte Carlo sensitivity analysis quantifies the robustness of the eLER metric to anaerobic digestion parameter uncertainty, and an explicit sensitivity analysis examines the eLER weighting scheme itself. All results are model-based; the integrated APV–AD system has not yet been field-demonstrated, which is identified as the principal direction for future experimental work. |  |
| Anahtar Kelimeler: (Keywords) Agrivoltaics; anaerobic digestion; extended land equivalent ratio; food–energy–water nexus; differential evolution; semi-arid climate; PAR saturation; 4E analysis |  |
| Tezin Amacı ve Önemi: (Objective and Significance of the Thesis) The overarching objective of this thesis is to develop and validate an integrated, climate-adaptive optimization framework that jointly designs bifacial agrivoltaic (APV) and anaerobic digestion (AD) systems, and to quantify their combined land-use, energy, water, and economic performance across contrasting climate zones. Three specific objectives follow from this: (1) to formulate a novel three-component extended Land Equivalent Ratio (eLER) that captures crop, electricity, and biogas outputs within a single, dimensionally consistent land-productivity metric, extending the classical two-component agrivoltaic LER; (2) to jointly optimize six APV and AD design variables via Differential Evolution across four climate zones spanning 12°N–48°N, identifying structural design principles (such as boundary convergence on row spacing) that generalize across climates; and (3) to evaluate the techno-economic and environmental viability of the integrated system through a 4E (Energetic, Economic, Energo-economic, Environmental) framework under multiple biogas valorisation scenarios. The significance of this work lies in addressing a systematic blind spot in the agrivoltaic literature: prior studies treat crop residues as waste, and prior AD–PV hybrid studies do not model agrivoltaic geometry or crop yield effects. No existing study provides a unified land-use metric, a jointly optimized design space, or a multi-climate comparison for integrated APV–AD systems. Given that semi-arid regions face the most acute simultaneous pressure on food, energy, and water resources, and given the accelerating global deployment of both agrivoltaics and on-farm biogas systems, a rigorously validated, climate-adaptive design framework has direct relevance for smallholder deployment planning, carbon-credit policy design, and food-energy-water nexus research. |  |
| Literatür İncelemesi: (Literature Review) Agrivoltaics was first proposed by Goetzberger and Zastrow (1982) and experimentally validated by Dupraz et al. (2011), who introduced the Land Equivalent Ratio (LER) as the standard dual land-use metric, reporting values of 1.35–1.73. Subsequent field studies (Marrou et al.; Barron-Gafford et al.; Elamri et al.) established the food-energy-water co-benefits of APV shading, while Riaz et al. (2021) and Mazzeo et al. (2025) advanced crop-specific and bifacial-optimized designs, reaching published two-component LER values up to 1.94. However, this literature consistently treats the organic crop-residue stream as waste, severing the link to the biogas economy. Anaerobic digestion (AD) research (Lindorfer et al.; Pilarski and Pilarska; Lallement et al.; Feng et al.) has separately characterized biochemical methane potential (BMP), Arrhenius temperature response, and digester thermal management, but AD studies that include a PV component (Pal and Ilango; Temiz et al.) treat the solar array as a generic electricity source without agrivoltaic geometry or crop modelling. The first study to couple APV and AD optimization, Nik Zad (2026), was limited to a single Mediterranean site without crop yield modelling or a land-use efficiency metric. This thesis addresses six identified research gaps: the absence of (G1) a unified three-output land-use metric, (G2) joint APV–AD design optimization, (G3) multi-climate comparative analysis, (G4) systematic characterization of thermal coupling across climates, (G5) a corrected growing-season water-savings metric, and (G6) modelling of the digestate-to-crop nutrient feedback (deferred to future field work). Full positioning against prior work is provided in Table 1 of the accompanying manuscript. |  |
| Tezin Yöntemi: (Methodology of the Thesis) The methodology couples six physical sub-models, solar geometry and inter-row shading, bifacial photovoltaic energy, tomato crop yield (via a photosynthetically active radiation, PAR, integral method with a crop-specific saturation threshold), evapotranspiration reduction, anaerobic digestion with Arrhenius temperature correction, and digester thermal balance, driven by hourly PVGIS-SARAH3 Typical Meteorological Year data for four climate zones (Konya, Almería, Ouagadougou, Freiburg). Six design variables (panel tilt, row spacing, module height, digester volume, hydraulic retention time, and PV heating fraction) are jointly optimized per site using Differential Evolution (population 90, 50 generations, L-BFGS-B local polishing) to maximize the proposed three-component eLER objective. A techno-economic 4E framework (Energetic, Economic, Energo-economic, Environmental) evaluates the optimized designs under seven AD valorization scenarios (baseline through carbon-credit), computing NPV, IRR, payback period, levelized cost of electricity, and CO₂ avoidance. Model validation combines two component-level checks against independent literature datasets (crop yield response to shading; BMP temperature response) with an external validation of the solar irradiance model against the pvlib library. Robustness is further assessed via a 500-draw Monte Carlo sensitivity analysis over literature-reported ranges for AD parameters (BMP reference value, capture efficiency, volatile-solids fractions), and via an explicit sensitivity analysis of the eLER component-weighting scheme against energy-content- and revenue-weighted alternatives, including a bi-objective (NSGA-II) Pareto analysis of the eLER–water-savings trade-off. The full simulation pipeline, six corrections made during model development, and all validation artefacts are version-controlled and are reported in the accompanying manuscript and its supplementary documentation. |  |
| Çalışma Planı: (Research Plan) İş-Zaman Çizelgesi (Project Timeline) İş No (Task No); Yapılacak İşler(Tasks); Aylar(Months). 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12. 1; Literature review, research gap analysis, and formulation of the eLER framework and six-model methodology; X; X; X. 2; Development, implementation, and unit/regression testing of the coupled simulation pipeline (solar geometry, POA, PV, crop yield, ET reduction, AD); X; X; X; X. 3; Multi-site Differential Evolution optimization (four climate zones) and AD/eLER-weighting sensitivity analyses (Monte Carlo, Pareto trade-off); X; X; X; X. 4; 4E techno-economic analysis, external validation (pvlib, literature benchmarks), and results interpretation; X; X; X; X. 5; Thesis writing, manuscript preparation, advisor review, and defense preparation; X; X; X; X. The 12-month research plan below reflects the standard structure of the work: framework development and validation, multi-site optimization and sensitivity analysis, and results synthesis / thesis writing. (Substantial preliminary work, model development, four-site optimization, and external validation, has already been completed at the time of this proposal; the schedule below reflects the full nominal plan for completeness and reference.) |
| Kaynaklar: (References) Allen, R.G. et al. (1998). Crop Evapotranspiration: Guidelines for Computing Crop Water Requirements. FAO Irrigation and Drainage Paper 56. Barron-Gafford, G.A. et al. (2019). Agrivoltaics provide mutual benefits across the food–energy–water nexus in drylands. Nature Sustainability, 2, 848–855. Duffie, J.A. and Beckman, W.A. (2013). Solar Engineering of Thermal Processes (4th ed.). Wiley. Dupraz, C. et al. (2011). Combining solar photovoltaic panels and food crops for optimising land use: Towards new agrivoltaic schemes. Renewable Energy, 36(10), 2725–2732. Faiman, D. (2008). Assessing the outdoor operating temperature of photovoltaic modules. Progress in Photovoltaics, 16(4), 307–315. Feng, L. et al. Biochemical methane potential (BMP) of vinegar residue and the influence of feed-to-inoculum ratios on biogas production. BioResources, 8(2), 2487–2498. Hargreaves, G.H. and Samani, Z.A. (1985). Reference crop evapotranspiration from temperature. Applied Engineering in Agriculture, 1(2), 96–99. Huld, T., Müller, R. and Gambardella, A. (2012). A new solar radiation database for estimating PV performance in Europe and Africa. Solar Energy, 86(6), 1803–1815. IRENA (2022). Renewable Power Generation Costs in 2022. International Renewable Energy Agency. Lallement, A. et al. A detailed database of the chemical properties and methane potential of biomasses covering a large range of common agricultural biogas plant feedstocks. Waste, 1(1), 195–227. Mazzeo, D. et al. (2025). Optimizing agrivoltaic systems: A comprehensive analysis of design, crop productivity and energy performance in open-field configurations. Mead, R. and Willey, R.W. (1980). The concept of a ‘land equivalent ratio’ and advantages in yields from intercropping. Experimental Agriculture, 16(3), 217–228. Mohammedi, S. et al. (2023). The impact of agrivoltaic systems on tomato crop: A case study in southern Italy. Processes, 11(12), 3370. Nik Zad, A. et al. (2026). Techno-economic optimization of agrivoltaic-powered anaerobic digestion plant for biomethane production. Energy Conversion and Management, 318, 119213. Pal, A. and Ilango, G.S. (2023). Design and techno-economic analysis of an off-grid integrated PV-biogas system with a constant temperature digester for a cost-effective rural application. Energy, 287, 129671. Pilarski, K. and Pilarska, A.A. (2025). Kinetics and energy yield in anaerobic digestion: Effects of substrate composition and fundamental operating conditions. Energies, 18(23), 6262. Riaz, M.H. et al. (2021). Crop-specific optimization of bifacial PV arrays for agrivoltaic food-energy production: The light-productivity-factor approach. IEEE Journal of Photovoltaics, 12(2), 572–580. Storn, R. and Price, K. Differential evolution — a simple and efficient adaptive scheme for global optimization over continuous spaces. Temiz, M., Sinbuathong, N. and Dincer, I. Development and assessment of a new agrivoltaic-biogas energy system for sustainable communities. |  |
| Etik Kurul İzin Belgesi Beyanı: ☐ Bu çalışma için Etik Kurul iznine gerek vardır. 16 Şubat 2017 PERŞEMBE Resmi Gazete Sayı: 29981 ERCİYES ÜNİVERSİTESİ LİSANSÜSTÜ EĞİTİM VE ÖĞRETİM YÖNETMELİĞİ Derslerin ve ders sorumlularının belirlenmesi MADDE 6 –(3) (Değişik: RG-17/07/2022-31895) Tezli yüksek lisans ve doktora/sanatta yeterlik programlarındaki öğrenciler için tez önerisinin kabul edildiği tarihi takip eden dönem başından itibaren uzmanlık alan dersi ve tez çalışması dersi açılabilir. Ancak etik kurul kararı ve kurum izni gerektiren tez çalışmalarında tezli yüksek lisans ve doktora öğrencileri dönem başında ‘Etik Kurul Kararı Bekliyorum’ seçeneğini işaretleyerek kayıtlarını yaptırabilirler. Etik kurul kararı/kurum izinleri alındıktan sonra tez önerisinin Enstitü Yönetim Kurulu kararıyla onaylanmasından sonra uzmanlık alan dersi ile tez çalışması derslerinin ataması yapılır. *Bu derslerin hangi tarihe kadar atanacağına dönem başında Enstitü Yönetim Kurulu karar verir. Uzmanlık alan dersi ile tez çalışması dersi yarıyıl ve yaz tatillerinde de devam eder. Uzmanlık alan dersleri dönemlik 5 AKTS kredisi olarak değerlendirilir. Her öğrenci, birinci tez danışmanı tarafından yüksek lisans programları için açılan uzmanlık alan dersi 1, 2 ve tez çalışması dersi 1, 2’yi, doktora programları için açılan uzmanlık alan dersi 1, 2, 3, 4 ve tez çalışması dersi 1, 2, 3, 4’ü almak ve geçmek zorundadır. Uzmanlık alan dersi bir öğrenci için en çok dört saat olmak üzere haftada azami 8 teorik ders saati, tez çalışması dersi, her bir tez öğrencisi için azami haftada bir pratik ders saati olmak üzere açılır. (*Dönem başında hangi tarihe kadar atanacağına dair Yönetim Kurulu Kararı Anabilim Dalı Başkanlıklarına EBYS ile bildirilecektir.) Ek: Etik kurul tarafından düzenlenen izin belgesi ekte sunulmalıdır. Belge alınmadı ise Etik kurul izin belgesi alınmasına yönelik yapılan talep, kanıtlarıyla (Dosya Numarası vb.) ekte sunulmalıdır. İzin belgesi alındıktan sonra teslim edilmelidir. ☒ Bu çalışma için Etik Kurul iznine gerek yoktur. Notlar: 1.Etik Kurul İzin Belgesi beyanının uygun ve doğru şekilde yapıldığına dair sorumluluk, Yükseköğretim Kurumları Bilimsel Araştırma ve Yayın Etiği Yönergesi kapsamında öğrenci ve danışmanındır. 2. Bilimsel araştırma ve yayın etiğine aykırı eylemlerde bulunduğu tespit edilen kişiler hakkında Yükseköğretim Kurumları Bilimsel Araştırma ve Yayın Etiği Yönergesi Madde 2-(1)-b, 10-(1) ve 10-(4) uyarınca, gerekli disiplin işlemleri ile diğer idari, hukuki ve cezai işlemler Yükseköğretim Kurumu tarafından yürütülür. 3.Tez çalışması için gerekli Etik Kurul izni Tez çalışmasına başlamadan önce Danışman rehberliğinde kurallara uygun bir şekilde öğrenci tarafından alınacaktır. Öğrenci, Etik Kurul tarafından verilen izne bağlı kalacaktır. 4.Etik Kurul izni gerekli Tez çalışması ile ilgili değişikliklerde, Etik Kurul bilgilendirilecektir. Bildirim sorumluluğu öğrenciye aittir. |  |
| Onay: Amdou MAIGA Prof. Dr. Gamze GENÇ (Name Surname, Signature,Date) (Name Surname, Signature,Date) Öğrenci Danışman (Student) (Advisor) |  |

TEZ ÖNERİSİ HAK SAHİPLİĞİ FORMU Tez Önerisi’nde 5846 sayılı Fikir ve Sanat Eserlerinin 10. maddesi uyarınca kişilerin Tez önerisi üzerinde Ortak Hak Sahipliği, Paylı Hak Sahipliği ve/veya Hak Sahipliği Feragat formu düzenlenmelidir. Tez önerisi üzerinde hakkı olan kişilerin belirlenebilmesi için Tez Önerisi Formunda bulunan kişilerin (danışman, araştırmacı “lisansüstü öğrencisi”) "Ortak Hak Sahipliği" ya da “Paylı Hak Sahipliği” Veya “Hak Sahipliği Feragat” beyan formunu doldurup imzalaması gerekmektedir. Tez önerisinin bölümlerinin arasında kesin sınırlarla paylara ayrıldığı durumlarda PAYLI, ayrılamadığı durumlarda ise ORTAK fikri hak sahipliği tercih edilmelidir. ☐ Tez gerekçesi Önerisinde Ortak Hak Sahipliği vardır. ☐ *Tez gerekçesi Önerisinde Paylı Hak Sahipliği vardır. ☐ Tez Önerisinde Danışman olarak adım geçmekle birlikte, Tez Önerisi üzerinde hiçbir Hak Sahipliğim (Fikri Katkım) yoktur. ☐ Tez Önerisinde Öğrenci olarak adım geçmekle birlikte, Tez Önerisi üzerinde hiçbir Hak Sahipliğim (Fikri Katkım) yoktur. *Paylı Hak Sahipliğinin tercih edildiği durumda "Paylı Hak Sahipliği Beyan Formunda" hak sahipliği beyan edilen her bir kişi için Tez Önerisinin hangi bölümlerinde payı olduğu belirtilmelidir. *Pay Hak Sahipliği Talep Edilen Bölümler. Danışman (Advisor). Öğrenci (Student). Amdou MAIGA Prof. Dr. Gamze GENÇ (Name Surname, Signature,Date) (Name Surname, Signature,Date) Öğrenci Danışman (Student) (Advisor)